MA6

| No. | INSTRUCTIONS FOR UPPER DIE INSPECTION |  | KEY POINTS / CONTACTS |
| --- | --- | --- | --- |
| 1. | - Ensure that the die area is free from hazards by clearing out the die area before transporting the die to the workshop. - You can not work under the die if it is not supported by the crane and the stands. |  | ENSURE PERSONNAL PROTECTIVE EQUIPMENT (PPE) IS WORN: - 1. Hard hat with chin strap 2. Steel cap boots 3. Gloves. 4. Torch. 5. Beware of sharp edges. |
| 2. | Place die on rubbers evenly on the die area. Refer to MAW 68 for die loading setting and turning. |  |  |
| 3. | Put up sign. (DIE CHECK IN PROGRESS) |  | . |
| 4. | Open die with reference to MAW 68. |  |  |

| No. | INSTRUCTIONS |  | KEY POINTS / CONTACTS |
| --- | --- | --- | --- |
| 5. | Place your lockout on crane remote, ensure the crane is supporting the die on the stand. Remove the crane remote battery and keep with you. |  |  |
| 6. | CASTING: Visually inspect for any cracks. |  |  |
| 7. | BLADE: Visually check for any chips Tighten Bolts Use an impact Wrench (set @ No. 2) |  | . Example of a chip |
| 8. | EJECT PIN: Loosen housing Check for proper performance (push in, must return) |  | If found faulty, replace with lock tight. |

| No. | INSTRUCTIONS |  | KEY POINTS / CONTACTS |
| --- | --- | --- | --- |
| 9. | SLIDE PLATES: Check for scratch marks and wear. Apply grease. |  | Wipe away old grease before inspection. Use gloves. |
| 10. | GUIDE POST BUSHES: Check for scratch marks and wear. Check that the bushes are secured properly Apply grease. |  | Wipe away old grease before inspection. Use a torch. |
| 11. | LIFTER RUBBERS: Tighten mounting bolts Check rubbers for cracks |  | . Example of damaged rubber |
| 12. | NITROGEN GAS: Check that the gauge shows correct pressure, 1500 MPA. Use soap water to check for leaks. Applicable only on DIE 1,18,19 and 9 |  | Check all connectors for leaks |

| No. | INSTRUCTIONS FOR LOWER DIE INSPECTION |  | KEY POINTS / CONTACTS |
| --- | --- | --- | --- |
| 13. | CASTING: Visually inspect for any cracks |  | Check that the 30mm bolts that holds the lower die to the 300mm base are all tight. Die 18 and 19 only |
| 14. | BLADE: Visually check for any chips Tighten Bolts Use an impact Wrench (set @ No. 2) |  | Example of a chip |
| 15. | GUIDE POSTS Check for scratch marks Apply grease |  | Wipe away old grease before inspection. Use gloves. |
| 16. | LIFTER Check for skid bar scratch marks Tighten bolts Check springs |  | Press down the lifter and check that it returns to its up position. If it does not, the spring may be broken |
| 17. | GUIDE ROLLER Check that bearings spin freely Tighten mounting bolts |  |  |
| 18. | SLIDE PLATES: Check for scratch marks Apply grease |  | Wipe away old grease before inspection. Use gloves. |
| 19. | FRONT GUIDE Check for scratch marks Tighten mounting bolts |  | Spin to check that the bearings are free. |
| 20. | CHUTES Check for deformation and damage Tighten mounting bolts |  | Ensure all rollers are working correctly. |
| 21. | INSIDE DIE Check and clean |  | Check for any scrap, rags, bolts or tools that may have been left behind during maintenance. Use a torch. |
| 22. | UNDER DIE | Place die on tressels and remove any burrs underneath the die. | NB: Do not remove crane slings when working under the die. |
| 23. | Close the die With reference to MAW 68. |  |  |
| 24. | Clean up die area after die service, (PROPER 5S) |  | Ensure that there is no tools or items left behind when the inspection is complete |
| 25. | Place back battery and return the remote to the remote station or operator and remove sign. |  |  |
| 26. | Update maintenance connection and attach pictures of findings. |  | All findings and work done must be recorded on maintenance connection |